| ## Example robots moves in 1D corridor ## xt​=robot position ## Models: ## Motion model ## Robot moves +1 m each step with Gaussian noise: ## xt​=xt−1​+1+ϵ ϵ∼N(0,0.22) ## Sensor model: Robot measures distance to a wall at position 5: ## zt​=5−xt​+δ, δ∼N(0,0.52) ## Likelihood: |  | ## Step 0 — Initial belief Assume uniform belief in corridor [0,4]. Particles: |  | Step 1 — Prediction (motion update) motion model (example draws): ## Assume noise samples: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## New particles: represent predicted belief ## Bel^(x1) | Step 2 — Measurement update Sensor reading: z1=1.0 ## Expected measurement from particle x: z^=5−x Compute likelihood for each particle. Gaussian likelihood: ## omit constant since weights normalized later | ## Particle 1 ## x=1.6, z^=5−1.6=3.4 error=1.0−3.4=−2.4 ## Same way for particle 2 – 5 => sum S ## S=9.9e−6+0.00022+0.278+0.98+0.546≈1.804 => Step 3 normalized |  |  |  |  |
| ## Step 4 Posterior approximation Distribution heavily concentrated near: ## x ≈ 3.9, 4.55 ??? |  | # Step 5 — Resampling Draw 5 particles according to weights. | sample uniform numbers:[0.12, 0.3, 0.6, 0.75, 0.9] Select: 0.12 → particle 3 0.3 → particle 4 0.6 → particle 4 0.75 → particle 5 0.9 → particle 5 New particle set: [3.2, 3.9, 3.9, 4.55, 4.55] Weight reset w=1/5 |  |  |
| # Step 6 — Final belief after one iteration Posterior now concentrated near true position. ## Mean estimate x^=1/5(3.2+3.9+3.9+4.55+4.55)=4.02 ## Robot likely around x~4 |  |  |  |  |  |  |
| ## answer requested theory items ## 1.Effective sample size Neff​ [0.178,0.323,0.337,0.102,0.060] ## Neff​ = 3.8 Since N=5, ## particle diversity reduced → resampling justified. |  | # 2. Second iteration interpretation Observation sequence shrinks uncertainty: - t=0: uniform - t=1: cluster near 4 - t=2: tight cluster near 4.9 |  |  | Particles numerically approximate this integral |  |
| ## 3. Compare numerically with EKF ## Posterior here is skewed [4.1,4.95,4.9,4.9,5.65] |  | Distribution: - asymmetric - heavy tail right EKF would approximate with mean≈4.9 and variance small → loses asymmetry and multimodality. Particle filter preserves non-Gaussian shape. |  |  |  |  |
| # 4. Degeneracy without resampling If resampling skipped: weights propagate multiplicatively: After several steps: - one particle weight → 1 - others → 0 Example here: largest weight = 0.337 already dominates. After few steps: Neff​→1 Filter collapses → poor approximation. Resampling redistributes particles to high-probability regions. |  | # 5. Optimal proposal q(xt∣xt−1,zt) General SIS weight: ## Variance minimized when: ## Using Bayes ## This uses measurement during sampling, not only weighting |  |  |  |  |
| This uses measurement during sampling, not only weighting. Advantages: - lower variance weights - fewer particles needed Disadvantage: - sampling from p(xt∣xt−1,zt) often analytically hard Hence standard PF uses simpler proposal: |  | # Big picture after full two-step example We have numerically demonstrated: - Bayes recursion implemented with samples - Prediction ≈ sampling motion - Correction ≈ likelihood weighting - Degeneracy appears naturally - Resampling stabilizes filter - Posterior converges toward robot position This is the full operational mechanics of Monte Carlo Bayes filtering in robotics |  |  |  |  |

| ## Example Particle filter ## 1. Problem Setup (Formal Notation) Grid World: 5 positions [A, B, C, D, E] , discrete state space X={A,B,C,D,E} ### Motion model (State Transition): But because the space is discrete and we have only “right” or “stay”: For the rightmost position E, attempting to move right → stay (since no cell beyond). ### Observation Model: Let zt∈{Wall,Open} ### Initial Belief: bel0​(x)=p(x0​)=Uniform(X)=1/5​ for each x ### Particle Representation: We use N = 4 particles. Each particle is a state hypothesis Initially: Notice: even true prior is 1/5, we approximate with 4 particles of equal weight Initial particle set (random draw from prior): [A, C, E, B] | ## 2. Step 0: Initialization (t=0) ### We have 4 particles x0i, drown i.i.d.from S initially uniform: Belief approximation: This gives probability 0.25 to states {A, C, E, B} and 0 to D. This is an approximation of the true uniform prior. ## Let in t=0 we are in state C |
| --- | --- |
| ### 3.1 step 1. Time t=1 (control and observation) ### NIS Only (No Resampling) ## Receive Control u1="move right" ## Observation z1=Open (true robot state is C → moves to D, so D is Open) ## For each particle, apply motion model: ## random no. 0.3 > 0.25 go right ## Particles after motion: [B, D, E, B] ## Weights remain unchanged before observation: still w0[i]=0.25 | ### 3.2: Observation update (weighting) ### than normalize Calculate likelihood p(z1="Open" ¦ position): ## Update unnormalized weights: wi = wi × p(z¦xi) Sum = 0. 225 225 + 0. + 0.025 + 0.225 = 0.700 |
| ### 3.2: Normalize Weights | Details: ## Particle set after NIS update |
| ### 3.3: Effective Sample Size (Degeneracy Measure) ### NIS Output & Analysis State estimate: Weighted average - B: 0.321 + 0.321 = 0.642 weight - D: 0.321 weight - E: 0.036 weight - Estimate = (0.642×B + 0.321×D + 0.036×E) = Between B and D Effective Sample Size (ESS): ESS Neff= 1/Σ(wi²) = 1/(0.321² + 0.321² + 0.036² + 0.321²) = 1 / (0.103 + 0.103 + 0.001 + 0.103) = 1 / 0.310 = 3.23 Observation: We started with 4 particles, ESS=3.23 → some degeneracy already! (mild degeneracy) | ### 3.4 : State Estimation (Weighted Mean) ## weighted histogram ## or for continuous approximation we might compute a weighted ## average of positions. Here we can compute the most likely state ## (maximum weight particle) or weighted average if we assign numerical ## values to positions (A=1, B=2, C=3, D=4, E=5). ## Maximum weight particle: Particles at B and D have weight 0.3214, B appears twice, D once. ## The highest weight value is at B (two particles with combined weight 0.6428). ## So the MAP estimate would be B. Weighted average (numerical): Let positions map to numbers: A=1, B=2, C=3, D=4, E=5. ? ## x^t=2×0.3214+4×0.3214+5×0.0357+2×0.3214 ## =0.6428+1.2856+0.1785+0.6428=2.7497 (between B and C) ? |
| ## 4. Step1 continued: Add Naive Resampling - Construct a categorical distribution q over particles with probabilities w1[i]​. - Sample N indices ik∼q with replacement. - New particle set: x1new[k]​=x1[ik​]​ - Reset weights: w1new[k]​=1/N1 | ### 4.1: Build Cumulative Distribution for Resampling From t=1 weights: |
| ### 4.1: Resample N=4 particles Draw 4 random numbers: [0.15, 0.55, 0.70, 0.95] - 0.15 < 0.321 → Select Particle 1 (B) - 0.55 between 0.321 and 0.642 → Select Particle 2 (D) - 0.70 between 0.678 and 1.000 → Select Particle 4 (B) - 0.95 between 0.678 and 1.000 → Select Particle 4 (B) Resampled particles: [B, D, B, B] Reset weights: All become w= 0.25 | ### 4.2 Low-Variance Resampling (Systematic) Draw r∼Uniform[0,1/N] - For k=1,…,N - Select particle j such that cumulative weight at j−1 < uk​ ≤ cumulative weight at j. For r=0.3 and u1​=0.10, u2​=0.35, u3​=0.60, u4​=0.85 Mapping 0.10 -> B, 0.35 -> D 0.60 still D, 0.85 -> B Result [B,D,D,B] E still lost. Low-variance ensures that any particle with weight > 1/N (here 0.25) is guaranteed at least one copy. E's weight 0.0357 < 0.25, so it is not guaranteed and may be lost. If we had a different r, it might be kept, but probability is low. |
| ### The state transition probability is |  |
| ### 5. Step 2 Time t=2 (Repeat Process) Use 4.1 [B, D, B, B] all weights 0.25 Apply motion model stay 0.5, move right 0.5 Draw random numbers | ### 3.3: Apply Motion (u="move right") |
| ### 3.4: Receive Observation z="Open" (True position is D → Open) Update weights: | ### 3.5: Normalize All weights = 0.225/0.900 = 0.25 Wait! All particles have equal likelihood, so weights stay uniform! ESS = 4.0 (perfect) ## Step 4: Time t=3 - Low-Variance Resampling ### 4.1: Start from uniform weights [0.25, 0.25, 0.25, 0.25] Cumulative: [0.25, 0.50, 0.75, 1.00] |
| ### 4.2: Low-Variance Resampling Step A: Pick random start r ∈ [0, 1/N] = [0, 0.25] ## Let r = 0.1 Step B: Generate N points: u_i = r + (i-1)/N - u₁ = 0.10 - u₂ = 0.10 + 0.25 = 0.35 - u₃ = 0.35 + 0.25 = 0.60 - u₄ = 0.60 + 0.25 = 0.85 | ## Step C: Map to particles: |
| ### Observation: With uniform weights, low-variance gives same result as random. The power shows when weights vary! | ## Step 5: Demonstrate Low-Variance Advantage Let's simulate a degenerate case from Step 1: Scenario: After NIS, weights were [0.321, 0.321, 0.036, 0.321] Cumulative: [0.321, 0.642, 0.678, 1.000] |
| ### 5.1: Naive Resampling (with random draws [0.05, 0.35, 0.65, 0.99]) - 0.05 < 0.321 → Particle 1 (B) - 0.35 between 0.321 and 0.642 → Particle 2 (D) - 0.65 between 0.642 and 0.678 → Particle 3 (E) ← Low weight particle selected! - 0.99 between 0.678 and 1.000 → Particle 4 (B) ### Result: [B, D, E, B] → E preserved (low weight) | ### 5.2: Low-Variance Resampling (r=0.1) u₁=0.10 → [0, 0.321] → Particle 1 (B) u₂=0.35 → [0.321, 0.642] → Particle 2 (D) u₃=0.60 → [0.642, 0.678] → Particle 3 (E) u₄=0.85 → [0.678, 1.000] → Particle 4 (B) ## --- --- Same result! But let's try r=0.3: u₁=0.30 → [0, 0.321] → Particle 1 (B) u₂=0.55 → [0.321, 0.642] → Particle 2 (D) u₃=0.80 → [0.678, 1.000] → Particle 4 (B) ← Skips E! u₄=1.05 → Wraps to 0.05 → [0, 0.321] → Particle 1 (B) Result: [B, D, B, B] → E lost ## Key insight: Low-variance guarantees any particle with weight > 1/N will be selected at least once. Here, E had weight 0.036 < 0.25, so it's not guaranteed! |
| ## Step 6: KLD-Sampling Demonstration (Conceptual) ne e predavano KLD From Step 1 particles: [B, D, E, B] Step A: Create histogram bins (bin size = 1 position): - A: 0 particles - B: 2 particles (weight 0.642) - C: 0 particles - D: 1 particle (weight 0.321) - E: 1 particle (weight 0.036) | Step B: Compute KL divergence from uniform: ??? Uniform: [0.2, 0.2, 0.2, 0.2, 0.2] Empirical: [0, 0.642, 0, 0.321, 0.036] (normalize: divide by 1.0) Actually, we use histogram of particle counts: [0, 2, 0, 1, 1] Empirical distribution: [0, 0.5, 0, 0.25, 0.25] (normalized by N=4) KL = Σ pi log(pi/qi) = 0×log(0/0.2) + 0.5×log(0.5/0.2) + 0×log(0/0.2) + 0.25×log(0.25/0.2) + 0.25×log(0.25/0.2) = 0 + 0.5×log(2.5) + 0 + 0.25×log(1.25) + 0.25×log(1.25) = 0.5×0.916 + 0.25×0.223 + 0.25×0.223 = 0.458 + 0.056 + 0.056 = 0.570 |
| Step C: From Fox 2003 formula: k = number of non-empty bins = 3 ε = 0.05 (desired accuracy) Required particles = (k-1)/(2ε) = 2/(0.1) = 20 particles! Conclusion: With only 4 particles, KL divergence is high (0.57). We need ~20 particles to represent this distribution well. This shows why adaptive particle counts help! |  |
| ## 🎯 Key Learning Points: - NIS degeneracy: After 1 step, ESS dropped from 4.0 to 3.23 - Resampling helps: Brought ESS back to 4.0 - Diversity loss: Resampling lost particle E (was already low weight) - Low-variance: More systematic, less random than naive resampling - KLD insight: 4 particles insufficient for this distribution | ## KLD stands for Kullback-Leibler Divergence (also called Kullback-Leibler Distance). |