Localization

Review the particle filter

Finite number of particles – every particle is representing a potential position of the robot

Belief is represented by a set of particles

Robot takes action u and then observes z. w = 1/N

Update particles

Comparison to Kalman filters: both work in continuous. Where would we choose particle over kf: if we have no idea where we might be. Kf is good for tracking, with some noise. Particle starts with full uncertainty. Particle filters are scalable, more interactions lead to better solution.

Where kf is better than particle: gaussian noise and dynamics are linear or can be linearized. Efficient. Exact inference with fixed computation.

Why switching from a particle filter to Kalman wouldn’t work

Localization:

X refers to the robots pose. Pose is the only thing we are interested in.

M is the robots map of the environment.

The motion model depends on previous state map and action.

State and transition model are no longer attempting to capture all relevant factors.

Markov localization: taking any base filter and applying it to the localization problem.

Pose is usually not sensed

Localization is done with respect to a global coordinate system

Much easier to measure within its own coordinate system.

Odometry – measurement of the robot’s change in pose.

Multiple measurement models: to integrate into the framework of bayes filters, we will assume conditional independence given the current robot pose and map.

Localization taxonomy: position tracking is probably the easiest localization problem. Estimate robot’s current pose given observations controls and knowledge of the initial state.

Global localization: estimate the robots current pose given observations and controls.

Ekf localization -

Monte carlo: useful if we get lost